Upgrade firmware operation manual一、Search device

Prepare a network cable, connect the RJ45 interface of the switcher to the LAN, open the upgrade software provided by “Devicewell”, and click Search Device. The device name and IP address will appear as shown in the figure below;

Device model IP address display

- upgrade the firmware

Device information display

Click the upgrade button

Drag the firmware here

Continue with the above steps, double-click the device model you want to upgrade in the left device column, drag the firmware provided by "Devicewell" into the upgrade tool, click the upgrade button, the device will automatically upgrade, and the software will display upgrade after the upgrade is successful Success; as shown in the figure below (note that the device cannot be powered off and the network cable is unplugged during the upgrade process);

- What should I do if the software cannot search for the device

The first method: first check whether the IP setting of the switcher device is set to dynamic IP acquisition (DHCP), operate the network setting function in the menu through the knob; if the switcher has been set up for dynamic IP acquisition, connect the LAN cable to the switcher , Your LAN will automatically assign an IP address to the switcher. At this time, you can search for the device again;

as shown in the figure below:

1.The second method: directly connect the switcher to the computer with a network cable, first set the switcher to the static IP function, and use the knob to operate the network setting function in the menu;；

2.Fix the switcher to an IP address, for example, fix the switcher to 192.168.1.20, and set the computer to a fixed IP address；

3.Steps for setting the IP address of the computer, open the Network and Sharing Center → click Local Area Connection → click Properties → double-click Internet Protocol Version 4 (TCP IPv4), an IP address setting page will appear, check the use of the following IP address in it, and fill it in manually IP address 192.168.1.30, manually fill in the subnet mask 255.255.255.0, and finally click OK。

3.1.The third method: confirm whether the device IP address is connected to the local area network normally, there is a Windos start menu in the lower left corner of the computer desktop, click start, a search box will pop up, enter cmd and press enter, a dos command line will pop up, also It is the so-called console, enter the IP address of the device you want to search for, such as ping 192.168.1.20 and click the enter key; (note that you need to press the space bar after entering ping)

Enter CMD